GUIDELINES — For projects that allow existing pavement to be open cut.  Include in the proposal: SP314-000110-00or SP314-000100-00. {2007-S302B00}

SP302-000100-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

RESTORING EXISTING PAVEMENT

July 12, 2016

I.	DESCRIPTION

This work shall consist of restoring existing pavement, removed for installation or repair of utilities such as, but not limited to pipe culverts, conduits, water and sanitary sewer items.

II.	MATERIALS

Asphalt Concrete shall conform to Section 211 of the Specifications.

Aggregate Subbase material shall conform to Section 208 of the Specifications.

Asphalt Material shall conform to Section 210 of the Specifications.

Fine Aggregate shall conform to Section 202 of the Specifications.

Coarse Aggregate for surface treatment shall conform to Section 203 of the Specifications.

Hydraulic Cement Concrete Class A3 shall conform to Section 217 of the Specifications.

Steel Reinforcement shall conform to Section 223 of the Specifications.

III.	PROCEDURES

Pavement restoration shall be according to this Provision and plan notes.

Backfill shall be according to Section 302.03(a)2.g. of the Specifications.

Asphalt Concrete shall be placed and compacted according to Section 315 of the Specifications.

Surface Treatment shall be placed according to the special provision for Asphalt Surface Treatment and the attached drawing.

Concrete Pavement shall be placed according to the special provision for Patching Hydraulic Cement Concrete Pavement and this special provision.  Open trench in Hydraulic Cement Concrete Pavement should be located at existing transverse joints if at all possible.  If concrete pavement is removed within two feet of an existing transverse joint, pavement removal shall be extended two feet beyond the joint.  Reinforcing steel and dowels shall be installed according to Road and Bridge Standard PR-2.  Joint replacement shall be according to Road and Bridge Standard PR-2.

IV.	MEASUREMENT AND PAYMENT

Restoring Existing Pavement unless otherwise specified will not be measured for separate payment, the cost thereof shall be included in the price bid for the utility to which it pertains according to Section 302.04, Section 520.06 or Section 700.05 of the Specifications, as appropriate.  However, widths and depths in excess of the attached drawing that are authorized or directed by the Engineer will be paid for according to Section 109.05 of the Specifications.

| NOTES: The following methods for restoring existing pavement shall be adhered to unless otherwise specified on the plans. 1. Pipe culverts, conduits and utility items shall be installed according to the applicable Road and Bridge Standards and Specifications. 2. Subbase - Aggregate material Type 1, Size 21A or 21B. 3. Asphalt Concrete Type BM-25.0 7” Min 4. Surface - Asphalt Concrete Type SM-9.5D @ 165 lbs. per sq. yd. |  | 5. Surface - Blotted Seal Coat Type C: The initial seal and final seal shall be CRS-2, CMA-2 or CMS-2h liquid asphalt material @ 0.17 gal./sq. yd. with 15 lbs. of No. 8P stone/sq. yd. each. The blot seal shall be CRS-2, CMS-2 or CMS-2h liquid asphalt material @ 0.15 gal./sq. yd. with 10 lbs. of fine aggregate grade B sand per sq. yd. 6. Subbase - Aggregate material Type 1 Size 21B 7. Surface - Hydraulic Cement Concrete, high early strength, matching existing structure for depth and surface texture. |
| --- | --- | --- |