GUIDELINES - For use on all projects that use Prefabricated Vertical Drains (wick-drains).

SP302-000300-01

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

PREFABRICATED VERTICAL DRAINS

January 8, 2018

- DESCRIPTION

The work shall consist of furnishing and installing prefabricated vertical drains (PVDs) in accordance with the Plans, Specifications and this Special Provision.  The drains shall consist of a nominally 4-inch wide, band-shaped plastic core enclosed in a geotextile fabric-wrapped jacket and shall be spaced and arranged as shown on the plans or as directed by the Engineer.

- DEFINITIONS

All terms and definitions outlined in ASTM D4439 shall apply in this specification.

- MATERIALS

  - PVD

PVDs shall be currently listed on Materials Division Approved List No. 39.

The PVD shall be newly manufactured materials and consist of a polyethylene or polypropylene drainage core enclosed in or integrated with a geotextile jacket.  The geotextile jacket shall be made of non-woven fabric and continuous filaments of 100 percent polypropylene or polyethylene.  The strip drain or its components shall have the physical characteristics outlined below.

    - The geotextile jacket shall allow free passage of pore water to the core without loss of soil material or piping.

    - The core shall provide continuous vertical drainage paths.

    - The PVD shall be capable of resisting all bending, punching, compression, and tensile forces imposed during install and during the design life of the drain without damage and so that the discharge capacity is not adversely affected.

    - Anchors shall be free of rust and sharp or jagged edges.

    - Anchor plates, if used, shall not exceed 8 inches in the maximum horizontal dimension.

    - The geotextile jacket material shall conform to the following minimum specifications, which shall be shown by testing provided for every 1.5 million linear feet of product, the test results being traceable to the product on which they were run and samples of the tested product retained for one year from test date, and tested either by GAI-LAP certified laboratories certified in the individual tests, or by laboratories verified via split sampling at least once every three years with such GAI-LAP certified labs.

| Test Item | Requirement Designation | Minimum Average Roll Value |
| --- | --- | --- |
| Grab Tensile Strength | ASTM D4631 | 120 lbs |
| Trapezoidal Tear Strength | ASTM D4533 | 50 lbs |
| Puncture Strength | ASTM D4833 | 38 lbs |
| Elongation at Break | ASTM D4632 | >50% |
| Permittivity | ASTM D4491 | 0.45 sec-1 |
| AOS | ASTM D4751 | <= #70 sieve |

    - The core shall be continuous polyethylene or polypropylene plastic material fabricated to promote drainage along the axis of the PVD, and shall conform to the following minimum specifications:

| Test Item | Requirement Designation | Minimum Average Roll Value |
| --- | --- | --- |
| Grab tensile strength on full drain width | ASTM D4595 | 225 lbs |

    - The mechanical properties (strength and modulus) of the assembled PVD shall equal or exceed those specified for the component geotextile jacket and core.

    - The assembled drain shall be resistant against wet rot, mildew, bacterial action, insects, salts in solution in the groundwater, acids, alkalis, solvents, and any other significant ingredients in the site groundwater for three years.

    - One single type of assembled PVD shall be used on the project unless otherwise specified or approved by the Engineer.  The assembled drain shall have the following minimum properties:

| Test Item | Requirement Designation | Minimum Average Roll Value |
| --- | --- | --- |
| Discharge Capacity | ASTM D4716 @43.5 psi @25% compression | >= 1gpm >=1 gpm |

  - DELIVERY, STORAGE, AND HANDLING OF MATERIAL

    - PVD materials shall be labeled or tagged in such a manner that the information for sample identification and other quality control purposes can be read from the label.  As a minimum, each roll shall be identified by the manufacturer as to lot or control numbers, individual roll number, date of manufacture manufacturer, and project identification on the jacket and core.

    - During shipment and storage, the PVD shall be wrapped in heavy paper burlap, plastic, or a similar heavy-duty protective covering.  The PVD shall be protected from sunlight, dirt, debris, and other detrimental substances during shipping and on-site storage.

    - All material which is damaged during shipping, unloading, storage, or handling; or which does not meet the minimum requirements of the PVD material will be rejected by the Engineer.  No payment will be made for rejected material.

- QUALIFICATIONS

  - CONTRACTOR QUALIFICATIONS

The Contractor shall have a minimum of five years of experience with the installation of PVDs and shall have completed three or more PVD installations of similar size and complexity within the last five years.  Provide at least one superintendent and one operator for the PVD equipment each with a minimum of five years of experience with the equipment and with PVD installation.  Alternatively, the Contractor shall provide detailed information on the training and experience of any superintendents or operators with less than five years but at least two years of experience for approval by the Engineer.

  - SUBMITTALS

The Contractor shall submit proposed actions and schedule to address any deficiencies within five business days after receiving a list of deficiencies.  The Contractor shall allow 15 calendar days for the Engineer to evaluate and approve all submissions.  The Contractor shall submit required data and state which PVD product they intend to install at least 21 calendar days prior to the start of PVD installation.  Approval by the Engineer will not relieve the Contractor of its responsibilities to provide material and equipment necessary to install the PVDs in accordance with the plans and specifications.  If the method of installation does not produce a satisfactory drain, the Contractor shall alter its method or equipment as necessary to comply with the Plans and Specifications.

    - Qualifying Experience

The Contractor shall submit proof of three or more projects of similar size and complexity on which they have installed PVDs within the last five years.  Identify the projects by project name, location, project description, length of the PVDs installed, linear footage of the PVDs installed, completion date, geotechnical consultant, owner/designer, and contract manager.  Provide phone numbers and business addresses for the owner/designer, geotechnical consultant, and contract manager.  Present the following information for each project listed as a reference:

      - Project name, location and completion date

      - Surface and subsurface conditions

      - PVD installation equipment and technique

      - Minimum, maximum, and average rates of PVD installation

      - Cost and duration of PVD installation

      - Average length of PVD installed and total linear footage of PVD installed

      - Current phone numbers and business addresses for the owner/designer, geotechnical consultant, and contract manager

    - Schedule

The Contractor shall provide written notice of intended schedule to Engineer at least 21 days prior to the installation of any PVDs.

    - Installation Procedures

The Contractor shall submit to the Engineer details of the sequence and method of installation.  The submittal shall, at a minimum, contain the following specific information concerning the scheduled work.

      - Size, type, weight, maximum pushing force, actual pushing force for anticipated set-up, track bearing pressure, and the configuration of the installation rig.

      - Shop drawings showing PVD layout and identification numbers.

      - Dimensions, weight, materials, and length of the mandrel.

      - Details of PVD anchorages that are anticipated.

      - Means of determining the depth of the advancing drain at any given time and the length of the drain installed at each location.

      - Detailed description of proposed installation procedures.

      - Estimated minimum, maximum, and average rates of PVD installation.

      - Proposed methods for penetrating existing embankment fill or subsurface obstructions.

      - Proposed plan for constructing a level working surface for installation of PVDs.

      - Manufacturer’s literature on PVD installation.

    - Certification of PVD Properties and Handling

Provide manufacturer’s Certification that delivered materials, identified by lot number, meet specification requirements, prior to delivery of PVDs.  Provide supplier’s Certification that delivered material, identified by lot number, has been properly stored away from sunlight, moisture, and dirt.

    - Samples of PVDs

Submit 3 samples of the PVD and 3 samples of any proposed splices.  Each sample shall be at least 5 feet long.  Samples of spliced PVD shall be long enough to include the splice plus 2 feet of unspliced drain on both sides of the splice.

    - Installation Records

Submit proposed installation record forms including date and time of inspection, PVD identification number, installed length, length of pre-augering or spudding, description of obstruction or other interruptions, daily quantity summary, total quantity summary, and other contract summary items.  Interim reports shall be submitted weekly.  A complete copy shall be submitted within two weeks after completion of PVD installation.

  - PROJECT CONDITIONS

    - Prior to bidding, the Contractor shall visit and examine the work site and all conditions thereon and take into consideration all such conditions that may affect this work, in accordance with Section 102.04 of the Standard Specifications.  Any subsurface data obtained by VDOT available for review may be obtained by contacting the District Materials Office.

    - Protection of Existing Structures:  Protect structures, underground utilities, and other construction from damage by PVD installation.

- EXECUTION

  - EQUIPMENT

    - Equipment shall be of a type that will cause a minimum of disturbance of the subsoil during the installation operation and shall maintain the mandrel in a vertical position.

    - PVDs shall be installed using a mandrel or sleeve that shall be inserted (pushed) into the soil.  The mandrel or sleeve shall protect the drain material from tears, cuts, and abrasions during installation, and shall be retracted after each drain is installed.

    - The mandrel or sleeve shall be straight and shall be sufficiently stiff to prevent wobble or deflection during installation.

    - To minimize disturbance of the subsoil, the mandrel, sleeve, and shoe shall have a maximum cross-section area of 12.1 sq. in.

    - When requested by the Contractor and approved by the Engineer, a maximum 8-inch diameter auger/drill/spud/etc. shall be used to pre-drill or spud through obstructions and/or hard and dense materials.

    - The mandrel or sleeve shall be provided with an anchor rod, plate, or similar arrangement at the bottom to permit the installation of the drain and to anchor the drain tip at the required depth at the time of mandrel withdrawal.  The dimensions of the anchor shall conform as closely as possible to the dimensions of the mandrel so as to minimize soil disturbance.  The Engineer will determine the acceptability of the anchorage system and installation procedure.

    - The choice of anchoring system is the responsibility of the Contractor, but it shall be the smallest surface area required to secure the PVD.

  - TRIAL INSTALLATIONS

    - Prior to installation of the PVDs within the required areas, the Contractor shall demonstrate that its equipment, method, and materials produce a satisfactory installation.  The Contractor shall provide the trial PVDs at locations designated by the Engineer.  Approximately 4 trial PVDs shall be installed.  The Engineer may also choose trial areas to determine the need for pre-augering prior to production installation.  Additional trial PVDs shall be installed, as required by the Engineer, or as approved by the Engineer when significant changes in the subsurface conditions or installation are encountered by the Contractor.

    - All trial drains provided to the satisfaction of the Engineer will be measured and paid for as Prefabricated Vertical Drains.

  - INSTALLATION

    - If, at any time, the approved method does not produce satisfactory PVDs, the Contractor shall alter his method or equipment as necessary to comply with these Specifications at no additional expense to the Department.

    - The Contractor shall provide a level (less than 2% grade change) work area of at least 25 feet wide.

    - PVDs shall be located, numbered, and staked by the Contractor, relative to a baseline and benchmark provided by the Engineer.  The Contractor shall take all reasonable precautions to preserve the stakes and is responsible for any re-staking.  The as-installed location of the PVDs shall not vary by more than 12 inches from the plan locations designated on the drawings.

    - PVDs shall be installed from the working surface to the elevation shown on the drawings, or to such elevation as directed by the Engineer.  The Engineer may vary the depths, spacing, or the number of drains to be installed, and may revise the plan limits for this work as necessary.

    - During PVD installation, the Contractor shall provide the Engineer with suitable means of determining the depth of the advancing PVD at any given time and length of the drain installed at each location.

    - PVDs shall be installed using either constant load or constant rate of advancement methods, together known as “static methods.”  A vibrator shall be available for use only in areas where the constant load or constant rate of advancement methods cannot install the PVDs by using the full static force available to the mandrel to the design elevations.  PVD which cannot be installed to the design penetration using only static methods shall be advanced with the use of the vibrator, and will be considered for compensation.  The vibrator shall only be used in cases where design penetration cannot be achieved.

    - PVDs that are more than 12 inches from the design plan location or are damaged or improperly installed will be rejected and abandoned in place and will not be measured for payment.  The Contractor shall install additional wick drains to replace unacceptable PVDs.

    - Where PVDs are to be installed through the existing embankments or through hard and dense materials, the Contractor shall pre-drill or spud through the obstructions prior to installation of the PVD, as required by site conditions and with approval of the Engineer.

    - A PVD shall be abandoned before reaching the design penetration when the rate of installation is less than 6 inches per second with the full static force and maximum vibrator output.  The use of falling weight impact hammers is not allowed.

    - Equipment for testing PVDs shall be plumbed prior to installing each drain and shall not deviate from the vertical more than 5 inches in 10 feet during installation of any drain.

    - Installation techniques requiring driving or jetting will not be permitted.  The injection of limited amounts of water shall be allowed to facilitate anchoring of the PVDs.

    - The installation shall be performed without any damage to the drain during advance or retraction of the mandrel.  In no case will alternate raising or lowering of the mandrel during advance be permitted.  Raising the mandrel will only be permitted after completing the drain installation.

    - There shall be a 12-inch section of the PVD material protruding above the ground surface at each PVD location.  The PVD material shall be cut neatly at its upper end at the working grade, or as specified in the contract drawings.

    - Where obstructions are found below the working surface, which cannot be penetrated using normal and accepted procedures, the Contractor shall complete the drain from the elevation of the obstruction of the working surface and notify the Engineer.

    - The Contractor shall observe precautions necessary for protection of any existing utilities, structures, and field instrumentation devices.  The Contractor shall pay for replacement of any instrumentation equipment that has been damaged or becomes unreliable as a result of his operations.

  - PRE-AUGERING/OBSTRUCTIONS

    - The Contractor shall be responsible for penetrating overlying fill material as necessary to satisfactorily install the PVDs.  Satisfactory installation may require clearing obstructions defined as any man-made or natural object or strata that prevents the proper insertion of the mandrel and installation of the PVD.

    - The Contractor may use augering, spudding, or other approved methods to loosen the soil and any obstruction material prior to the installation of the PVDs. The obstruction clearance procedure is subject to approval of the Engineer.

    - When obstructions are found, the Contractor shall immediately notify the Engineer prior to completing the drain and prior to installing any other drains.  The Contractor shall attempt to install drains adjacent to the obstructed location, in the presence of the Engineer.  Based upon the results of these installations and at the direction of the Engineer, the Contractor shall:

      - Attempt to install an offset drain within 18 inches horizontally of the obstructed drain; or

      - Implement obstruction clearance procedures and install the drain at the design location.  Obstruction clearance procedures shall be used only as directed by the Engineer.

  - SPLICING

    - Splicing of PVD material shall be done by stapling to insure structural and hydraulic continuity of the drain.

    - The jacket and core shall be overlapped by a minimum of 6 inches at any splice.

    - A maximum of one splice per drain installed will be permitted without specific permission from the Engineer.

- MEASUREMENT AND PAYMENT

Prefabricated Vertical Drains (PVD) will be measured in linear feet and will be paid for at the Contract linear foot price, measured to the nearest linear foot from the ground surface to the installed depth of the anchor, complete in place.  This price shall include grading the work area, furnishing, installing and splicing PVDs and all labor, materials, tools and equipment necessary to complete the work.

Trial drains installed as directed by the Engineer will be measured and paid for as Prefabricated Vertical Drains, even if they do not function as intended.

Obstructed PVDs will be paid for at the contract unit price for PVDs, as measured to the nearest foot from the bottom of the vertical strip drains (at the obstruction), to ground surface.

The length of the PVD extending above the ground surface will not be measured for payment.

Pre-Augering will be measured in linear feet and will be paid for at the Contract linear foot, measured from the bottom of the obstruction depth to the ground surface to the nearest foot.  This price shall include pre-drilling (augering, spudding, rotary air flush drilling, open hole with roller bits, hydraulic excavator with hydraulic hammer and pick, etc.) and removing or driving through (spudding) subsurface obstruction (dense natural soils or man-made fill materials, buried debris, etc.) to maximum depth of 15 feet below ground surface and all labor, materials, tools and equipment necessary to complete the work.

| PAY ITEM | PAY UNIT |
| --- | --- |
| Prefabricated Vertical Drains (PVD) | Linear feet |
| Pre-Augering | Linear feet |