GUIDELINES — Asphalt projects (plant mix only). {2007-S315QM1}

SP315-000300-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

LIMITS OF MAINLINE OVERLAY AT INTERSECTIONS TO PAVED ROADS

July 28, 2010; Reissued July 12, 2016

I.	DESCRIPTION

This work shall consist of furnishing and placing asphalt concrete overlay pavement courses on existing paved roadway surfaces that intersect the mainline roadway pavement overlay.  This work shall be performed according to the Special Provision for Placement of Asphalt Concrete Overlays, Sections 211 and 315 of the Specifications; and where Stone Matrix Asphalt (SMA) is specified in the Contract, Sections 248 and 317 of the Specifications; and as specified herein.  Where pavement planing is required it shall be performed according to Special Provision for Cold Planing (Milling) Asphalt Concrete Operations and Section 515 of the Specifications and as specified herein.

II.	MATERIALS

Materials shall be according to Section 211 of the Specifications; and where Stone Matrix Asphalt (SMA) is specified in the Contract, Section 248 of the Specifications; and the Special Provision for Placement of Asphalt Concrete Overlays.

III.	EQUIPMENT

Equipment for furnishing and placing asphalt concrete overlay shall be according to Section 315 of the Specifications; and where Stone Matrix Asphalt (SMA) is specified in the Contract, Section 317 of the Specifications.  Pavement planing equipment shall be according to the Special Provision for Cold Planing (Milling) Asphalt Concrete Operations and Section 515 of the Specifications.

IV.	PROCEDURES

Furnishing and placing asphalt concrete overlay shall be according to Section 315 of the Specifications; and where Stone Matrix Asphalt (SMA) is specified in the Contract, Section 317 of the Specifications.  Where pavement planing is required, it shall be according to the Special Provision for Cold Planing (Milling) Asphalt Concrete Operations and Section 515 of the Specifications and as specified herein.

The Contractor shall overlay the intersecting paved road from the edge of pavement of the mainline roadway pavement overlay to a point that includes the entire radius of the intersecting paved road according to the attached drawing.  This distance from the edge of pavement of the mainline roadway pavement overlay shall not exceed 50 feet measured according to the drawing herein.

On curb and gutter sections where planing is required for the mainline roadway overlay, planing shall also be required on the intersecting paved road area prior to these areas being overlaid.

Asphalt concrete overlay pavement placed on existing paved roadway surfaces that intersects the mainline roadway pavement overlay shall be constructed using a method approved by the Engineer, which shall include the cutting of a notch into the pavement.  The approved method shall provide a smooth transition between new pavement and existing pavement. Such tie-ins shall conform to Section 315.05(c) of the Specifications except that all joints at tie-in locations shall be tested using a 10-foot straightedge according to Section 315.07(a) of the Specifications.  The variation from the testing edge of the straightedge between any two contact points with the pavement surface shall not exceed 1/4 inch.

V.	MEASUREMENT AND PAYMENT

Overlay at intersections to paved roads will be measured and paid for according to the pay items of Section 315 of the Specifications; and where Stone Matrix Asphalt (SMA) is specified in the Contract, Section 317 of the Specifications, and the Special Provision for Cold Planing (Milling) Asphalt Concrete Operations and Section 515 of the Specifications.