GUIDELINES — For projects requiring dynamic pile testing for end bearing piles. {2007-S403C01}

SP403-000100-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

DYNAMIC PILE TESTING FOR END BEARING PILES (LRFD)

February 7, 2014; Reissued October 27, 2016_

I.	DESCRIPTION

This work shall consist of dynamic testing of piles by the use of electronic monitoring equipment, reprocessing the data and furnishing a written report of the results.

II.	EQUIPMENT

All equipment necessary for the dynamic monitoring such as gages, cables, etc. shall be furnished by the Dynamic Testing Consultant.  The equipment shall conform to the requirements of ASTM-4945-08, Standard Test Method for High Strain Dynamic Testing of Piles.

III.	PERSONNEL

The Contractor shall employ a Dynamic Testing Consultant to install or supervise the installation of the necessary equipment, to perform the dynamic monitoring and to prepare the Dynamic Testing Report.

The dynamic monitoring operator shall have a minimum of two years experience, at least one of which shall have been in data acquisition from high strain dynamic pile testing and successful performance on at least two projects in similar geotechnical conditions, or who has a Certificate of Testing: Basic Level or better on the Foundation QA Examination for Providers of Pile Dynamic Analyzer (PDA) Testing Services.

The Dynamic Pile Testing Report shall be prepared by a Registered Professional Engineer with a minimum of five years experience, at least two of which shall have been in data interpretation from high strain dynamic pile testing and successful completion of at least five projects in similar geotechnical conditions, or who has a Certificate of Interpretation: Advanced Level or better on the Foundation QA Examination for Providers of PDA Testing Services.

IV.	TESTING

Dynamic testing shall be conducted in the presence of the Engineer and during the entire time piles are initially driven or redriven and during pile restrike testing.

The Contractor shall notify the Engineer of the date and time for dynamic testing at least 48 hours prior to testing.  Such notice shall be given during the normal work hours of the Department.  If additional dynamic testing is ordered by the Engineer, the Contractor shall schedule the tests in cooperation with the availability of the Engineer.

Where possible, splices to the pile(s) shall be made prior to the start of driving so that dynamic testing can be performed without interruption.

The Contractor shall fasten a pair of transducers and a pair of accelerometers in place prior to testing.  Piles shall be driven until the soil resistance measured is equal to or greater than the Nominal Pile Resistance as measured during driving shown on the plans and the required minimum tip elevation and penetration have been obtained or as directed by the plans, approved wave equation analysis or as approved by the Engineer.  The Contractor shall remove the transducers and accelerometers after the dynamic testing is completed.

All signals resulting from initial testing and any restrike testing shall be recorded and made available upon the request of the Engineer.

V.	REPORTS

If requested by the Engineer, the following information shall be provided within 24 hours after completion of the testing: for each blow from the Dynamic Driving Records provide the Depth, Maximum Transferred Energy, Blows per Minute (include strokes, fuel settings, bounce chamber pressures, etc. as applicable), Maximum Tensile Stress, Maximum Compressive Stress and Pile Capacity.

The Contractor shall furnish the Engineer a Dynamic Pile Testing Report with the production pile order list.

The Dynamic Pile Testing Report shall include the following information for each pile tested:

Project identification and location

Location of test,

Date of test,

Description of the subsurface soil condition including log of nearest boring

Description of the test pile

Description of pile installation equipment, the lead type and any special installation equipment

Description of dynamic testing equipment, including model and software version(s) utilized in obtaining, evaluating and reporting dynamic data.

A copy of the Pile Driving Record

Pile Installation Details and Comments

Discussion of the hammer performance

Discussion of pile integrity

For at least every fifth blow from the Dynamic Driving Records: the Depth, Maximum Transferred Energy, Blows per Minute (including strokes, fuel settings, bounce chamber pressures, etc. as applicable), Maximum Tensile and Compressive Stress and Pile Capacity

A graphical presentation of the following: Pile Penetration versus Maximum Transferred Energy, Maximum Compressive Stress, Maximum Tension Stress and Mobilized Pile Capacity

The results from a signal-matching program that estimates static soil resistance and simulates static load test results including Mobilized Pile Capacity for the shaft and toe with the associated parameters used in the estimation

A summary tabulation of the following information: Pile Location and Designation, Date Driven, Pile Tip Elevation, Visual Blow Count Rate, Transferred Energy, Hammer Efficiency, Maximum Driving Stresses, Dynamic Testing Mobilized Pile Capacity, Signal-Matched Mobilized Pile Capacity for Shaft, Toe and Combined.

Recommendations for production pile driving criteria based on the results of the testing program. Driving criteria shall include: blow count to obtain the required Mobilized Pile Capacity (include: stroke(s), fuel setting(s), bounce chamber pressure(s), etc. as applicable), criteria for controlling driving stresses in the pile including maximum allowable hammer stroke to control driving stresses in the pile and criteria for terminating driving in the event of high blow court before reaching the approved tip elevation.  Pile driving criteria shall be approved by the Engineer.

VI.	MEASUREMENT AND PAYMENT

Dynamic pile testing (End Bearing) will be measured and paid for at the contract unit price per each, which price shall be full compensation for providing all services of the testing consultant and dynamic monitoring operator as specified herein including providing, installing, monitoring and removing the dynamic testing equipment, for providing the data and preparing the written documentation specified, and for all tools, labor, materials, and incidentals necessary to complete the work.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Dynamic Pile Test (End Bearing) | Each |