# Guidelines: Use when the project schedule does not allow adequate time for the  Department to perform the protected fish relocation within cofferdams. Determination should be made prior to the PAC meeting.

# SP522-000270-00

# VIRGINIA DEPARTMENT OF TRANSPORTATION

# Special Provision for

# PROTECTED Fish Species

Contractor Performs relocation

January 14, 2019

- Description

Stream Name is known to provide habitat for the Fish Common Name, a state or federally protected species. The Contractor shall take protective measures as described herein.

- Requirements

A Time of Year Restriction (TOYR) applies on all instream work from (DATE) to (DATE) with the exception of dewatering and other work within cofferdams, or as otherwise specified in the applicable water quality permits issued for the project.

The Contractor shall install and remove cofferdams outside of the TOYR.

The Contractor shall conduct the fish relocation work. Fish relocation shall be performed by an individual with a valid Virginia Department of Game and Inland Fisheries (VDGIF) Threatened and Endangered Species Permit. The Contractor shall notify the Engineer of the selected individual and provide a copy of the Threatened and Endangered Species Permit at least 14 days before beginning fish relocation, or a shorter timeframe as determined by the Engineer for subsequent fish relocations. The Engineer will not authorize fish relocation without receipt of the permit. Reporting under the permit is the responsibility of the approved individual doing the fish relocation work; the Contractor shall provide a copy of all reports to the Engineer. Dewatering shall not begin until the fish relocation is complete, and the Engineer provides written authorization for dewatering to proceed.

If any cofferdam has overtopped due to excessive stream flow after initial relocation efforts were completed, additional fish relocation shall be required before subsequent dewatering activities. Additional fish relocation shall follow the same requirements as the initial fish relocation.

- Measurement and Payment

The cost associated with this work shall be included in other appropriate items. No separate payment will be made for compliance with the requirements herein.