GUIDELINES – Asphalt projects (Plant Mix Only).

SP703-000110-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

REPLACEMENT OF LOOP DETECTORS

March 16, 2017

- Description

Loop Detectors shall be installed in accordance with the Section 703. Installation of loop detectors shall be performed in the presence of the Engineer.

- Materials

Loop detector cables shall be No. 14 AWG stranded copper in accordance with Section 238.  Loop sealant shall be from the Pre-Approved Traffic Control Device Listing.

New loop detectors shall be of the same size, configuration and locations as existing loop detector(s) unless otherwise indicated.

- Procedures

When replacement of loop detectors is included in the Contract, the Contractor will be required to install new loop detector items either within the planed surface prior to the placement of new surface mix, or through the finished riding surface.

Loop detectors shall be installed at the depth specified in the TD-Series Standard Drawings for loop detectors installed in the planed surface or final riding surface, as applicable.

When an existing loop detector is taken out of service by the Contractor’s planing operation, the Contractor shall have the new loop detector items installed and operational 96 hours after being taken out of service, unless otherwise stated in the Contract.  In no case shall any loop detector be out of service for more than 96 hours.  If the Contractor chooses to install new loop detector items through the final riding surface, all loop detector items shall be installed and operational within 96 hours after completion of the paving operations in the affected intersection.

The Contractor shall notify the Engineer at least 72 hours prior to planing at locations that contain loop detectors.