SQ704-000100-00

GUIDELINES – For projects where the final pavement markings will be installed by the Department or by other contracts.{2007-SU704001A}

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

PAVEMENT DOTTING

October 8, 2008; Reissued July 12, 2016_

I.	DESCRIPTION

This work shall consist of the furnishing and placing of pavement dots to establish the location of pavement markings on the roadway according to the requirements specified herein and as directed by the Engineer.  This work is for those sections of roadways where the final pavement markings will be installed by the Department or by other contracts.  Those sections of roadways where the Contractor installs the final pavement markings shall not require pavement dotting, however, premarking may be accomplished at the Contractor’s option according to Section 704.03 of the Specifications.

II.	MATERIALS

Pavement dots shall be removable tape (Type D, Class II) conforming to Section 246.03(g)1 of the Specifications.  Pavement dots shall consist of 4-inch by 4-inch squares or 4-inch diameter circles and shall be of the same color as the final pavement markings to be installed.

III.	PROCEDURES

Pavement dots shall be placed on the new pavement surface for each individual pavement marking line unless otherwise directed by the Engineer.  Pavement dots shall be placed in the same lateral position along the roadway where the existing markings were located.

Pavement dots shall be installed at 100-foot intervals in tangent sections and 50-foot intervals in curved sections.  Less spacing may be used as needed for but not limited to such pavement markings items as stop lines, crosswalk lines, and hatching.  Pavement dotting shall be installed according to the manufacturer's recommendation.

IV.	MEASUREMENT AND PAYMENT

Pavement dotting will be measured and paid for at the contract unit price per mile of pavement line dotted, to the nearest one-tenth of a mile.  This price shall be full compensation for furnishing and installing the pavement dots, and all materials, labor, tools, equipment and incidentals necessary to complete the work.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Pavement dotting | Mile |