Guidelines: For use on all projects.

SS200-002016-01	September 11, 2019

VIRGINIA DEPARTMENT OF TRANSPORTATION

2016 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 200 – GENERAL

SECTION 200 – GENERAL of the Specifications is amended as follows:

Section 200.04 – Acceptance Procedures for Aggregates is amended to replace the fourth paragraph with the following:

The No. 10 sieve shall be the dividing sieve for soils, select material, aggregate subbase material, and aggregate base material. The No. 8 sieve shall be the dividing sieve for asphalt concrete aggregates. That portion of the total aggregate retained on the sieves is defined as coarse aggregate, and that portion passing the sieves is defined as fine aggregate. Soundness tests will be performed according to the requirements of AASHTO T 104 without regard to these definitions of fine and coarse aggregate. Fine and coarse aggregates for hydraulic cement concrete are distinguishable by their conformity to the series of grading requirements specified in Sections 202 and 203, respectively, except that lightweight aggregate is specified in Section 206.

Section 200.06 – Technician and Batcher Certification is amended to replace the first paragraph with the following:

When the Contract requires a type of technician or batcher defined by this Section, the Contractor shall use a person certified by the Department. The Department will certify technicians and batchers upon a candidate’s satisfactory completion of an examination.

Section 200.06(m) – Soils and Aggregate Compaction Technician is inserted as follows:

Soils and Aggregate Compaction Technician: A Soils and Aggregate Compaction Technician conducts density, moisture content, and depth checks of soil placement and aggregate lifts during construction, including stabilized lifts. The Technician also monitors application rates of stabilizing chemicals used in soil and aggregate lifts in the field.