Guidelines: For use on all projects.

SS203-002016-01	September 19, 2019

VIRGINIA DEPARTMENT OF TRANSPORTATION

2016 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 203 – COARSE AGGREGATE

SECTION 203 – COARSE AGGREGATE is amended as follows:

Section 203.02 – Materials is amended by replacing the first paragraph with the following:

Coarse aggregate shall consist of crushed stone, crushed slag, crushed or uncrushed gravel, or lightweight aggregate. Coarse aggregate shall be clean, hard, tough, and durable pieces free from: adherent coatings and deleterious amounts of friable, thin, elongated, or laminated pieces; soluble salts; or organic materials.

Section 203.02(e) – Lightweight coarse aggregate is inserted as follows:

Lightweight coarse aggregate shall conform to Section 206.