GUIDELINES - For all projects.

SS238-002016-02	August 20, 2019

VIRGINIA DEPARTMENT OF TRANSPORTATION

2016 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 238 – ELECTRICAL AND SIGNAL COMPONENTS

SECTION 238 – ELECTRICAL AND SIGNAL COMPONENTS of the Specifications is amended as follows:

Section 238.02(d) – PE Conduit is replaced with the following:

PE Conduit shall conform to NEMA TC-7 for high-density PE duct except that the wall thickness of conduit with a diameter of 1-1/4 inches and less shall conform to UL-651 for heavy-wall PVC conduit. Conduit shall have a carbon black loading of 2.5 ±0.5 percent by weight in accordance with ASTM D1603.

Section 238.02(g)2a – Signal cables from the controller cabinet to signal heads is replaced with the following:

Signal cables from the controller cabinet to signal heads shall be minimum No. 14 AWG copper.

Section 238.02(g)2e – Pedestrian detection cables is inserted as follows:

Pedestrian detection cables shall be two conductor No. 14 AWG copper. IMSA 19-1 or 20-1 cables shall be used for aerial or duct installations. IMSA 19-5 or 20-5 cables shall be used for direct buried installations.

Section 238.02(h)7 – Backplates for signal heads is replaced with the following:

Backplates for signal heads may be either aluminum or aluminum composite, unless otherwise specified in the Plans. ABS plastic shall not be used.

Aluminum shall be 0.06-0.08 inch thick, smooth, flat, and free of metal burrs and splinters. Aluminum alloy shall conform to Section 229.

Aluminum Composite shall be a 0.07-0.09 inch thick aluminum plastic composite. The composite shall be manufactured by bonding two panels of aluminum to an extruded polyethylene core using a thermoset adhesive under tension and pressure in a continuous process. The interior coating of the aluminum panels shall be coated with an epoxy chromate primer for added bonding strength with the polyethylene.

- Black (non-reflective) Signal Backplates:

Aluminum black signal backplates shall be entirely powder coated black in accordance with manufacturer’s instructions.

For aluminum Composite black backplates, both sides of the aluminum composite exterior shall be entirely coated with black fluoropolymer paint.

- High-Visibility Signal Backplates (HVSBs) shall be preassembled by the manufacturer with a 3-inch retroreflective fluorescent yellow border on the outside of the front of the backplate.

All retroreflective sheeting on the front surface of the backplate shall be ASTM D4956, Type XI fluorescent yellow from the Materials Division’s Approved Products List 46, which shall be warrantied in accordance with Section 247.03.

Retroreflective sheeting shall be applied to the backplate with a zero-degree orientation (downweb direction perpendicular to the road) and adhered to the backplate in accordance with the retroreflective sheeting manufacturer’s instructions. Retroreflective sheeting shall be butt spliced when more than one piece of sheeting is adhered to the backplate. For each surface of the backplate, a maximum of five butt splices shall be used for three-section and four-section signal heads, and a maximum of nine butt splices shall be used for five-section signal heads.

Aluminum HVSBs shall be manufactured by covering the entire front surface with the retroreflective sheeting specified above, then subsequently applying black color to the front surface, except for the outer 3 inches of the front of the backplate which shall remain fluorescent yellow. The black color on the front surface of the backplate shall be obtained by screen printing or applying acrylic film. Black color on the back surface of Aluminum HVSB backplates shall be obtained by applying acrylic film to the aluminum or by screen printing on sheeting applied to the aluminum. Aluminum preparation and application of sheeting and film shall be in accordance with the sheeting manufacturer’s instructions.

Aluminum Composite HVSBs surfaces shall be coated with black fluoropolymer paint suitable for proper retroreflective sheeting adhesion. Retroreflective sheeting shall be applied to the outer 3 inches of the front surface of the backplate as specified above.

Section 238.02(h)20 – Photoelectric controls is replaced with the following:

Photoelectric controls shall be used at the electrical service control center for roadway lighting system or electrical service for overhead sign lighting structures to turn fixtures ON or OFF, typically in sync with dusk to dawn. Photoelectric controls shall conform to ANSI C136.10, and the following:

- Locking-type/twist-lock photoelectric control with locking-type receptacles

- Use a cadmium sulfide or filtered silicon type sensor

- Fail-on type so that the lights stay ‘ON’ if the photoelectric control fails to operate

- Factory preset and calibrated to turn on at 1.5 ± 0.5 footcandles (fc), not exceeding 3 fc

- Ratio of the turn-off to turn-on light level shall not exceed 1.65:1

- Output control relay shall have a time delay of 2 to 15 seconds to prevent false turn-off due to headlights and other transient light source.

- Normal operating voltage shall be in the range of 120V - 277V or 347V- 480V

- Capable of controlling load of up to1000 Watts for tungsten and 1800 volt-amps for ballast

- Have a built-in metal oxide varistor of minimum 160-Joules for surge/transient protection

- Operating temperature shall be in the range of -40ºF to 158ºF (-40ºC to 70ºC)

- Cover shall be an impact and ultraviolet-resistant material that complies with the flammability and impact requirements of UL-773

- Warranted against defects in materials and manufacture for at least 4 years.

All the receptacles at the electrical service control cabinets and LED fixtures (with the exception of individual sign lighting fixtures), shall be prewired 7-pin twist lock ANSI C136.41 complaint and shall come with rain-tight shorting cap.