GUIDELINES – For use on all projects with earthwork..

SS303-002016-04	March 14, 2019

VIRGINIA DEPARTMENT OF TRANSPORTATION

2016 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 303 – EARTHWORK

SECTION 303 – EARTHWORK of the Specifications is amended as follows:

Section 303.02(c) – Geotextile materials used for embankment stabilization is replaced with the following:

Geotextile materials used for embankment stabilization shall conform to Section 245.03(d).

Section 303.02(f) – Fabric used for Turbidity Curtains is inserted as follows:

Fabric used for Turbidity Curtains shall conform to Section 245.03(k).

Section 303.03(b) – Soil Stabilization is amended by replacing the first paragraph with the following:

Soil Stabilization: The Contractor shall begin soil stabilization as soon as practicable, but no later than the end of the next business day, following the day when the land-disturbing activities on any portion of the Project have permanently or temporarily ceased for an anticipated duration of greater than 14 days. The Contractor shall complete soil stabilization within seven days of reaching final grade or from when land-disturbing activities have permanently or temporarily ceased for an anticipated duration of greater than 14 days. Initiation of soil stabilization includes, but is not limited to, prepping the soil for vegetative or non-vegetative stabilization, applying mulch or other non-vegetative product to exposed soil, and seeding or planting the exposed area. The Contractor can initiate soil stabilization activities on a portion of the area to be stabilized and not on the entire area, so long as the initiation and completion of stabilization activities occurs on the entire disturbed area within the allowable timeframe for soil stabilization. Areas within 100 feet of the limits of ordinary high water or a delineated wetland are excluded from this requirement, but the work shall be continuously prosecuted until completed, and then stabilized immediately upon completion of the work in each impacted area. Soil stabilization includes: temporary and permanent seeding, riprap, aggregate, sod, mulching, and soil stabilization blankets and matting in conjunction with seeding. The applicable type of soil stabilization shall depend upon the location of areas requiring stabilization, time of year, weather conditions, and stage of construction operations.

Section 303.03(c) – Check Dams is replaced with the following:

Check Dams: The Contractor shall install required check dams at the interval and locations shown on the Plans in accordance with the standard specified. When the Plans do not provide locations or intervals the Contractor shall install check dams below the outfall end of drainage structures and other necessary locations in consultation with the Engineer and in accordance the check dam standard specified.

Temporary Check Dams conforming to Standard Drawing EC-16 for temporary erosion control shall conform to Section 244.02(o). Only products on the Department’s Approved List 83 may be used for temporary check dams. Substitutions for Standard Drawing EC-4 Rock Check Dams, Type I or II are not permitted.

Section 303.03(e)1 – Temporary Silt Fences is replaced with the following:

Temporary silt fences: Silt fences will be specified by type and shall be erected at locations shown on the Plans or as determined by the Engineer. Posts shall be driven no less than 24 inches into the ground uniformly installed with an inclination toward the potential silt load area of at least 2 degrees but not more than 20 degrees. Geotextile fabric used for silt fences shall be provided and erected at a height of 24 inches above original ground. The bottom of the fabric shall be entrenched in the ground 12 inches (6 inches vertically and 6 inches horizontally) in a minimum 6-inch by 6-inch trench. Silt fence may also be entrenched using a slicing method with a minimum of 8 inches sliced into the ground. A continuous roll of fabric cut to the length of the silt fence is preferred to avoid the use of joints. When joints are unavoidable, fabric shall be spliced together only at a support post, with a minimum 6-inch overlap, and securely sealed by double folding ends together. Attaching fabric to existing trees will not be permitted.

    - Type A silt fence usage is limited to a fill height of 20 feet or less. Posts shall not be spaced more than 4 feet apart and shall  be oak wood with nominal dimisions of 2 inch by 2 inch.

    - Type B silt fence is required for fill heights greater than 20 feet. Post shall not be spaced more than 4 feet apart and shall   be steel with a minimum weight of 1.25 pounds per linear foot.

Type A and B Silt Fence post shall have a finished height of no less than 6 inches above the fabric. The fabic shall be firmly secured to the post at both the top and vertical midpoint of the fabric. The top of the fabric shall be installed with a 1-inch tuck or reinforced top end section.

Type A and B Silt Fence shall be in accordance with the Standard Drawing EC-5.

Section 303.03(e)3 – Temporary Filter Barriers is deleted.

Section 303.03(g) – Erosion Control Mulch is amended by replacing the second paragraph with the following:

Mulch shall be applied to exposed slopes requiring mulch or to areas to be stabilized or paved within 48 hours after performance of grading operations in accordance with Section 603.03(e).

Section 303.03(i) – Turbidity Curtain is replaced with the following:

Turbidity Curtain: This work consists of installation, maintenance, and removal of a turbidity curtain, including all necessary cables, weights, and floats in accordance with this provision and in conformity with the lines, grades and details shown on the Plans or established by the Engineer. The curtain shall be provided as a temporary measure to minimize the drift of suspended material during construction of the Project.

Type I turbidity curtain shall be used in protected areas that are sheltered from waves; and exposed only to light winds, and to current velocities of less than one foot per second.

Type II turbidity curtain shall be used in areas subject to small to moderate current velocities (up to 2 knots or 3.5 feet per second) or moderate wind and wave action.

TYPE III turbidity curtain shall be used in areas subject to considerable current (up to 3 knots or 5 feet per second), tidal action, or where the curtain is potentially subject to wind and wave action.

In locations with currents greater than 3 knots (5 feet per second) perpendicular to the barrier, or weather conditions that cause a turbidity barrier to be ineffective, a turbidity barrier shall not be used.

Floatation shall be flexible, buoyant units contained in a floatation sleeve or collar attached to the curtain. Buoyancy provided by the floatation units shall be sufficient to support the required width of the curtain and maintain a freeboard of at least 3 inches above the water surface level, to a minimum of one foot above the bottom or a maximum ten foot depth at all stages of water levels.

Load lines shall be fabricated into the top and bottom of the curtain. The top load line shall consist of woven webbing or vinyl-sheathed steel cable and shall have a minimum break-strength of 9,800 pounds. The bottom load line shall consist of a chain incorporated into the bottom hem of the curtain of sufficient weight to serve as ballast to hold the curtain in a vertical position. Additional anchorage shall be provided if necessary to top load lines. The load lines shall have suitable devices, which develop the full breaking strength for connecting to load lines in adjacent sections.

The Contractor shall submit Working Drawings to the Engineer for review in accordance with Section 105.

The curtain shall be placed at the locations shown on the Plans and in accordance with the approved Working Drawings. The Contractor shall maintain the turbidity curtain in order to ensure the continuous protection of the waterway.

The curtain shall extend the entire depth of the watercourse whenever the watercourse is not subject to tidal action or significant wind or wave action.

In tidal or wind-and-wave action situations, the curtain shall never touch the bottom. A minimum 1-foot gap shall be established between the weighted lower end of the skirt and the bottom at the mean low water.

Turbidity curtains installed in a navigable waterway shall be marked with lighted buoys that conform to U.S. Coast Guard regulations.

When the curtain is no longer required as determined by the Engineer, the curtain and related components shall be removed in such a manner as to minimize turbidity. The curtain and related components shall become the property of the Contractor and shall be removed from the project.

Section 303.03(j) – Inlet Protection is inserted as follows:

When sediment retention rolls are used in accordance with the EC-6 standard for Type A or Type B inlet protection they shall conform to Section 244.02(o). Only sediment retention roll products on the Department’s Approved List 83 may be used for inlet protection Type A or B.

Section 303.04(i)1 – Settlement plates is amended to replace the third paragraph with the following:

Settlement plates shall remain in place until settlement has been completed as indicated by elevation readings taken by the Engineer at approximately 2-week intervals. Evaluation of the readings by the Engineer will be the final and sole governing factor for releasing embankments for grading operations. Upon written release by the Engineer, extensions of settlement plate pipe shall be removed to at least 2 feet below the subgrade, the pipe filled with lean grout, and the area backfilled and compacted. Other types of settlement gauges (e.g. Manometer Settlement Gauge) may be used as approved by the Engineer.

Section 303.06(e)4 – Check dams is replaced with the following:

Check dams (Rock Check and Temporary) will be measured in units of each and will be paid for at the contract unit price per each. This price shall include furnishing, excavating, constructing, maintaining and removing the check dams if, or when, no longer required.

Section 303.06(e)5 – Temporary silt fence type A is renamed Temporary silt fence type A or B and replaced with the following:

Temporary silt fence type A or B will be measured in linear feet from edge of the fabric to edge of fabric, complete-in-place, excluding laps, and will be paid for at the Contract linear foot price. Decomposed or ineffective geotextile fabric replaced after 6 months from the installation date will be measured in linear feet of temporary silt fence type A or B and paid for at 1/2 the contract unit price for temporary silt fence. Decomposed geotextile fabric required to be replaced prior to 6 months after installation will not be measured for payment. This price shall include furnishing, installing and maintaining the silt fence, including fabric and posts; removing and disposing of these materials, and dressing and stabilizing the area.

Section 303.06(e)6 – Temporary silt fence type B is deleted.

Section 303.06(e)8 – Temporary Filter Barriers is deleted.

Section 303.06(e)16c – Inlet Protection Type C is deleted.

Section 303.06(e)20 – Turbidity Curtain is replaced with the following:

Turbidity curtain will be measured in linear feet from edge of the curtain along the support cable. Turbidity curtain will be paid for at the contract unit price per linear foot for the type specified. This price shall include design details, furnishing, installing, maintaining, and removal of all materials necessary to complete the work.

Section 303.06 – Measurement and Payment is amended by revising the Pay Item Table as follows:

The following Pay Items are removed:

| Pay Item | Pay Unit |
| --- | --- |
| Check dam (Type) (Log, rock, or straw) | Each |

The following Pay Items are inserted:

| Pay Item | Pay Unit |
| --- | --- |
| Check dam (Type) (Temporary(Size) rock, or straw) | Each |