GUIDELINES — All projects with guardrail and w-beam median barriers.

SS505-002016-03	May 2, 2018

VIRGINIA DEPARTMENT OF TRANSPORTATION

2016 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 505 – GUARDRAIL AND W-BEAM MEDIAN BARRIERS

SECTION 505 – GUARDRAIL AND W-BEAM MEDIAN BARRIERS of the Specifications is amended as follows:

Section 505.03 – Procedures is amended to replace the thirteenth paragraph with the following:

Guardrail shall only be constructed by a guardrail erector included on the Department’s Approved List 12, who shall have a trained guardrail installer on the project during guardrail installation. For the purpose of this specification, a trained guardrail installer is a person who has a current certificate of training from a Department-approved guardrail installation training course.

Section 505.03 – Procedures is amended to replace the sixteenth paragraph with the following:

The Contractor shall submit two copies of the manufacturers’ recommended installation instructions and the FHWA NCHRP 350 or MASH approval letter for the type of new or salvaged guardrail end treatments being installed on the project to the Engineer at least 2 weeks before starting guardrail end terminal installation. All end terminals shall be from manufacturers on the Department’s Approved List 12 and the Department’s Approved List for NCHRP 350 or MASH products linked therein. New Type I Re-Directive Impact Attenuators and Guardrail Terminals shall be permanently identified by stamping or engraving in a location readily visible for inspection that is not susceptible to damage. The identification shall include Manufacturer, Date and Site of Manufacture, and Model Number.

Section 505.03(d) – Adjusting existing guardrail is amended by replacing the first paragraph with the following:

Adjusting existing guardrail beam shall consist of removing and disassembling the existing guardrail beam and offset blocks from the posts, drilling the post in accordance with the standard drawing, and reassembling the offset blocks and guardrail beam to the height required by current Standard Drawings or Specifications. Adjusting the existing guardrail beam shall be limited to 4 inches. Adjusting guardrail beam shall be limited to steel posts and shall be in accordance with the plan details and Standard Drawings. Adjusting existing guardrail beam will not be permitted within the pay limits of end terminals. The terminal shall be completely removed and reinstalled or a new terminal installed in accordance with the Standard Drawings and the manufacturer’s instructions.

Adjusting existing guardrail to meet the GR-MGS1 or GR-MGS1A Standard Drawings will not be permitted.

Section 505.04 – Measurement and Payment is amended by replacing the fifth paragraph with the following:

Terminal treatment or end anchorage for beam guardrail, cable guardrail, and steel median barriers terminating on the roadway side of the ditch line will be measured in units of each and will be paid for at the contract unit price per each.

Section 505.04 – Measurement and Payment is amended by inserting the following:

Guardrail height transition (Standard) will be measured in units of each and will be paid for at the Contract each price. This price shall include furnishing and placing posts, offset blocks, and all hardware necessary to fully install the height transition.

Section 505.04 – Measurement and Payment is amended by revising the Pay Item Table as follows:

The following pay items are inserted:

| Pay Item | Pay Unit |
| --- | --- |
| Guardrail end anchorage (Standard) | Each |
| Guardrail height transition (Standard) | Each |