SS701-002016-01	March 11, 2019

VIRGINIA DEPARTMENT OF TRANSPORTATION

2016 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 701—TRAFFIC SIGNS

SECTION 701—TRAFFIC SIGNS of the Specifications is amended as follows:

Section 701.02—Materials is replaced with the following:

(a)	Reflective sheeting shall conform to Section 247 and shall be selected from the Department’s Approved List 46. The color for the legend and background shall be in accordance with the MUTCD and as specified in the Plans.

- Overhead Permanent Signs (signs attached to sign structures which overhang travel lanes) that are not illuminated with sign lighting shall use ASTM D4956 Type XI reflective sheeting. Overhead permanent signs that are illuminated with sign lighting shall use ASTM D4956 Type IX sheeting.

- Non-Overhead Permanent Signs (including ground-mount signs, signs attached to traffic signal supports, and signs attached to sign structures that do not overhang travel lanes) shall use ASTM D4956 reflective sheeting as follows:

| Background Color of Sign | Sheeting Type1 |
| --- | --- |
| White | ASTM D4956 Type XI |
| Red | ASTM D4956 Type XI |
| Yellow | ASTM D4956 Type XI2 |
| Fluorescent Yellow-Green | ASTM D4956 Type XI3 |
| Green | ASTM D4956 Type XI |
| Black | ASTM D4956 Type XI |
| Purple | ASTM D4956 Type XI |
| Brown | ASTM D4956 Types IV, IX, or XI |
| Blue | ASTM D4956 Types IV, IX, or XI |
| 1The following signs may use ASTM D4956 Types IV, IX, or XI, regardless of color: Pushbutton education signs (R10-series signs mounted adjacent to pedestrian pushbuttons), Signs erected on bikeways physically separated from adjacent roads, R7- or R8-series parking restriction signs located on non-limited-access highways, D10-series Reference Location Signs (mile markers) and Intermediate Reference Location Signs, and Post-mounted street name signs. 2The yellow portions of all yellow W1-series (horizontal change of alignment) signs, W10-series (railroad warning) signs, and object markers, including supplemental plaques erected beneath those signs, shall be fluorescent. All other yellow sheeting on signs may be fluorescent or non-fluorescent. 3All temporary and permanent warning signs related to school zones, pedestrians, or bicyclists (including associated supplemental plaques) shall use fluorescent yellow-green sheeting where required by the VA Supplement to the MUTCD unless otherwise specified on the Plans. |  |

(b)	Sign panel rivets for overhead signs attached to cantilever, butterfly, or truss sign structures shall be powder coated to match the color of the portion of the sign sheeting from which the rivets will protrude. The rivets shall be fabricated in accordance with Standard Drawing SPD-1.

(c)	Sign panel substrates for permanent flat sheet signs shall be in accordance with the below table, and shall be smooth, flat, and free of metal burrs and splinters.

| Sign width1 | Allowable substrate |
| --- | --- |
| 47 inches and less | 0.080 or 0.100 inch aluminum |
| 48 inches and more | 0.100 inch aluminum |
| Overlay panel | 0.063 inch aluminum |
| 1For diamond-shaped signs, width is measured along the sign’s edge. |  |

Aluminum substrates for permanent flat sheet signs and overlay panels shall be aluminum alloy in accordance with Section 229.02(a).

(d)	Extruded sign panels shall conform to the Standard Drawings and Section 229.02(c).

(e)	Temporary signs shall conform to Section 512.02.

Section 701.03(a)2 Sign Panels is deleted.

Section 701.03(a)3 Applying retroreflective background sheeting is replaced with the following:

Applying retroreflective background sheeting: Sheeting shall be applied according to the manufacturer’s instructions and the detailed requirements herein.

The Contractor shall fabricate sign panels 16 square feet or less from a single piece of applied sheeting with no joints, splices, or laps, except that one factory splice from each roll is permitted..

When applying more than one width of reflective sheeting to a sign panel, sheeting edges shall form a clean vertical joint. Sheeting edges shall be applied in accordance with manufacturer's installation instructions to prevent edge lifting and allow for sheeting expansion under high temperature and humidity conditions.

The finished sign shall be free from cracks, gaps, streaks, wrinkles, blisters, discoloration, buckles, and warps and shall have a smooth surface of uniform color.

Section 701.03(a)4 Letters, numerals, arrows, symbols, borders, and other features of the sign message is amended to replace the first paragraph with the following:

Letters, numerals, arrows, symbols, borders, and other features of the sign message: Features of the sign message shall conform to the MUTCD and the Virginia Standard Highway Signs Book. Units of the sign message shall be formed to provide a continuous stroke width with smooth edges and a flat surface free from warps, blisters, wrinkles, burrs, and splinters. Features shall also conform to the following:

Section 701.03(b) Transportation and Storing Signs from the Fabricator is amended to replace the third paragraph with the following:

The Contractor may remove signs from storage and install them on their structural supports before the structure is erected; however signs and structural supports placed in the field prior to erection shall be supported and stored at a sufficient angle to facilitate water runoff from the sign while preventing the sign from coming in contact with the ground and preventing sign structure elements from sitting in standing water.

Section 701.03(d) Erection is amended to replace the first paragraph with the following:

Erection: The Contractor shall install sign panels on overhead sign structures with the required minimum and maximum vertical clearances as shown on Standard Drawing OSS-1, and with the lateral and vertical placement shown on the Plans.

Section 701.03(d) Erection is amended to replace the fifth paragraph with the following:

Vertical and horizontal spacing between installed signs shall be approximately 1 inch where multiple signs are installed on the same structure, unless shown otherwise on the Plans.

Section 701.03(d) Erection is amended to replace the thirteenth paragraph with the following:

Overlay panels shall be erected with aluminum rivets no less than 3/16 inch in diameter and of such length as to fasten the panels securely and form a compressed head conforming to the manufacturer’s recommendations. Rivets shall be located on 12-inch centers for 0.080 inch aluminum overlays and on 15-inch centers for 0.063 inch aluminum overlays, positioned 1 inch from each panel’s edges, completely around the sign’s perimeter. Where overlayment panels are 30 inches or greater in width, a column of rivets shall be installed on 12-inch centers for 0.080 inch aluminum overlays or 15-inch centers for 0.063 inch aluminum overlays down the centerline of the panel. Rivets shall be installed in such a sequence as to prevent buckling of the panels. When overlaying extruded aluminum signs, rivets shall be arranged to go through the flat part of the extrusion.

Section 701.03(f) Documentation Requirements is replaced with the following:

- Labels. All new permanent signs shall include fabrication labels, and a VDOT Identification Label. Labels may be made of either a self-adhesive, permanent weather resistant material or permanent sign material, and shall be a minimum 4-inch by 4-inch in size.

All information on such signs shall be indicated with sign ink or other permanent means capable of resisting weathering for the full duration of the sign sheeting warranty period, except that dates may be indicated with punching out of appropriate squares. All new signs shall be indicated “new” on the VDOT Identification Label.

Prior to applying the labels, the area shall be thoroughly cleaned to ensure proper adhesion or application of ink. Labels shall be placed on the back side of the sign panel in a location where they will not be obscured by sign supports or mounting hardware.

  - Fabrication labels. Labels provided by the sign fabricator that indicates sheeting manufacturer’s name or logo, sheeting product designations, lot numbers, sign fabricator’s name or logo, and month and year the sign was fabricated. All text and logos shall be at least 1 inch in height. For signs with multiple sheeting designations and/or multiple lot numbers, additional labels or supplemental labels are permitted.

  - VDOT Identification Label. VDOT’s standard 4.5-inch by 4.5-inch label shall be affixed to all new permanent traffic control device signs, as per the Virginia Supplement to the MUTCD.

- Inventory Sheet. The Contractor shall provide an .xlsx formatted file to the Engineer, using a sign inventory template provided by the Engineer. The file shall include the information required above for the label, as well as the following:

  - Route no.

  - Project UPC no. (if applicable).

  - Station or milepost information.

  - Lane designation.

  - MUTCD, if applicable and if denoted on the plans.

  - Sign message.

  - Sign width.

  - Sign height.

The cost of preparing and submitting the .xlsx formatted file shall be included with the cost of the sign panel pay items.

Section 701.04 Measurement and Payment is amended to replace the first paragraph with the following:

Sign panels will be measured in square feet and will be paid for at the Contract square foot price. This price shall include sign substrate, background sheeting, sign messages, finishing, framing units, hanger assemblies, bracing, stiffeners, splicing, backing strips, post clips/post clamps, warranty, and labeling.