Guidelines: For use on all projects with traffic signals.

SS703-002016-01	December 14, 2018

VIRGINIA DEPARTMENT OF TRANSPORTATION

SUPPLEMENTAL SPECIFICATION FOR

SECTION 703—TRAFFIC SIGNALS

SECTION 703—TRAFFIC SIGNALS of the Specifications is amended as follows:

Section 703.02(d) Signal Heads is amended to replace the fourth and fifth paragraphs with the following:

Backplates shall be included with all vehicle traffic control signal heads unless otherwise specified in the Contract. Backplates shall be specifically manufactured for the type and brand of traffic signal heads used or shall be of a universal design expressly manufactured for various types and brands of traffic signal heads. Backplates shall have a border width of 5 inches, shall be without louvers, and be of one-piece construction with the exception of those for five-section cluster signal heads, which may be a maximum of three pieces. All outside corners on backplates shall have a 3-inch radius.

Black Signal Backplates (both sides) and signal leveling attachments shall be flat black. Black signal backplates shall be aluminum or aluminum composite. Aluminum and aluminum composite shall conform to Section 238.

High-Visibility Signal Backplates (HVSBs) shall be provided if specified in the plans. HVSBs may be aluminum or aluminum composite; ABS plastic shall not be used. HVSBs shall be preassembled by the manufacturer in accordance with Section 238 of the Specifications.

Section 703.03(e)1a is replaced with the following:

Unless otherwise directed by the Engineer, backplates shall be attached with either bolts, washers, and lock nuts, or with self-tapping screws and washers.

The minimum number of fasteners connecting the backplate to the traffic signal head shall be four for each 12-inch traffic signal head section. Fasteners and all miscellaneous hardware shall be stainless steel unless otherwise directed by the Engineer. The fasteners shall be a minimum 3/16 inch diameter and 1/2 inch long.

When HVSBs are to be installed on new signal heads, cutting the backplate is not required unless otherwise directed by the Engineer.

Section 703.04—Measurement and Payment is amended to replace the thirteenth paragraph with the following:

Traffic Signal Head Section will be measured in units of each for the LED module size and backplate type specified and will be paid for at the Contract each price. This price shall include mountings, molded terminal block, visor, backplate, retroreflective sheeting (if required), fittings, realignments, and LED module.

Section 703.04—Measurement and Payment is amended by revising the Pay Item Table as follows:

The following pay item is removed:

| Pay Item | Pay Unit |
| --- | --- |
| Traffic signal head section (Size and type) | Each |

The following pay item is inserted:

| Pay Item | Pay Unit |
| --- | --- |
| Traffic Signal Head Section (LED module size, backplate type) | Each |