GUIDELINES — All metric unit projects only.  When this SP applies include the following in the proposal: cn100-00m026-03.{2007-SSMCON02}

SSCON-00M001-00	September 27, 2016

VIRGINIA DEPARTMENT OF TRANSPORTATION

2007 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

IMPERIAL UNIT TO METRIC UNIT CONVERSION

I.	GENERAL

This Supplemental Specification is included in this specific project because the Department has identified the units of measure to be used as metric units only.  The methods of conversion from imperial units into metric units required herein shall be used to convert only the specific units of measure in the Contract that are expressed only in imperial units.

Any imperial unit of measure in the Contract with an accompanying expression in a metric unit shall be referred to hereinafter as a “dual unit” measurement.  Such a “dual unit” measurement is typically expressed first in the imperial unit followed immediately to the right by the metric unit in parenthesis “()” or brackets “[]” where parenthesis is used in the sentence to convey other information.  Where “dual units” of measure appear in this project, only the metric units shall apply.  The imperial unit shown is not to be considered interchangeable and mathematically convertible to the metric unit and shall not be used as an alternate or conflicting measurement.

If conflicting measurements of the same item appear in two separate places in the Contract with one measurement being a solitary metric unit measurement expressed only as a “single unit” or as the second part of a “dual unit” (imperial/metric) measurement and, the conflicting measurement is a metric unit measurement converted from an imperial unit measurement using this Supplemental Specification; the metric unit measurement expressed as a “single unit” or as part of a “dual unit” shall apply.

Any obvious or apparent error or omission shall be corrected according to Section 105.12 of the Specifications.

The conversion methods herein shall apply to the following:

●	The Virginia Department of Transportation Road and Bridge Specifications book, dated 2016; and its accompanying revisions including all Supplemental Specifications, Special Provisions, and Special Provision Copied Notes in the Contract.

●	The Virginia Department of Transportation Road and Bridge Standards, dated 2016; and its accompanying revisions including all Standard sheets and Special Design drawings.

●	The 2011 edition of the Virginia Work Area Protection Manual with Revision Number 1 incorporated, dated April 1, 2015.

●	The 2009 edition of the MUTCD with Revision Numbers 1 and 2 incorporated, dated May 2012.

●	The 2011 edition of the Virginia Supplement to the MUTCD with Revision Number 1 dated September 30, 2013.

●	Other contract documents included in the Contract containing any use of imperial units of measure without an accompanying metric unit of measure.

The conversions herein shall not apply to the following:

●	That portion of any Supplemental Specification, Special Provision, Special Provision Copied Note, Standard sheet, Special Design drawing or other contract document included in the Contract where only a metric unit of measure is used.

●	That portion of any Supplemental Specification, Special Provision, Special Provision Copied Note, Standard sheet, Special Design drawing or other contract document included in the Contract where a “dual unit” (imperial/metric) measurement is used.

●	The plans, as defined in Section 101.02 of the Specifications.  Such plans are expressed only in metric units for this project.

●	The “Schedule of Items” in the Contract.  The quantities and units of measure of such items in the “Schedule of Items” are expressed only in metric units for this project.

II.	IMPERIAL UNIT TO METRIC UNIT CONVERSION

The table herein was developed as a tool for the specific purpose of converting measurements and units in the Contract expressed only as imperial units into the metric units required for this project.  Such conversions shall only apply to those measurements and units specifically designated in Section I of this special provision.

The units of measure, terms and abbreviations used in the table herein do not represent every conceivable imperial or metric unit of measure, term and abbreviation.  Such used in the table herein represents what typically appear in a broad range of Department projects.  The Contractor or Engineer may discover an imperial unit of measure, term and abbreviation in the Contract requiring conversion to metric according to Section I of this special provision that is not included in the table herein.  If such occurs the Contractor shall submit to the Engineer, for the Contractor’s approval, the method of conversion and its source, the metric unit and resulting measurement.

Rounding of converted measurements shall be performed in a manner that establishes metric unit measurements that best reflect the intent of the measurements originally expressed in imperial units.

When nominal imperial unit measurements, such as pipe or aggregate sizes, are converted; the resulting measurement shall be for the purpose of clearly identifying the corresponding nominal metric unit measurements for that item.

Any obvious or apparent error or omission in converting the units of measure from imperial to metric according to the table herein shall be corrected according to Section 105.12 of the Specifications.

Converting the units of measure from imperial to metric according to the requirements herein and of Section I of this special provision shall be performed according to the follow table:

TABLE OF IMPERIAL UNIT TO METRIC UNIT CONVERSION FORMULAS

| FROM IMPERIAL UNIT (IU) | FORMULA | TO METRIC UNIT (MU) |
| --- | --- | --- |
| DISTANCE | ============= | DISTANCE |
| Linear foot, ft | IU x .3048 = MU | Meter, m |
| Inch, in | IU x 25.4 = MU | Millimeter, mm |
| Microinches | IU x .0254 = MU | Micrometer, µm |
| Microinches | IU x 0.0000254 = MU | Millimeter, mm |
| Mile, mi (U.S. Statute) | IU x 1.60934 = MU | Kilometer, km |
| Station (100 feet) | IU x .3048 = MU | Station (100 meters) |
| Yard, yd | IU x 0.9144 = MU | Meter, m |
| DISTANCE (THICKNESS) | ============= | DISTANCE (THICKNESS) |
| Mil (.001 inch) | IU x .0000254 = MU | Meter, m |
| Mil (.001 inch) | IU x 0.0254 = MU | Millimeter, mm |
| Mil (.001 inch) | IU x 25.4 = MU | Micrometer, Micron (.000001 meter), µm |
| SLOPE RATE | ============= | SLOPE RATE |
| Inch per foot, in/ft | IU x 25.4 x 3.2808399 = MU | Millimeters per meter, mm/m |
| SLOPE RATE (AVERAGE) | ============= | SLOPE RATE (AVERAGE) |
| Inches per mile International Roughness Index (IRI) | IU x 25.4 x 0.62137 = MU | Millimeters per Kilometer, mm/km International Roughness Index (IRI) |
| AREA | ============= | AREA |
| Acre, ac | IU x 0. 0.4046873 = MU | Hectare, ha |
| Square foot, ft2 | IU x 92903.04 = MU | Square millimeter, mm2 |
| Square foot, ft2 | IU x .092903 = MU | Square meter, m2 |
| Square inch, in2 | IU x 6.4516= MU | Square centimeter, cm2 |
| Square mile, mi2 | IU x 2.589988= MU | Square kilometer, km2 |
| Square yard, yd2 | IU x 0.8361274= MU | Square meter, m2 |
| Square yard, yd2 | IU x 8361.274= MU | Square centimeter, cm2 |
| Unit (1,000 square feet) | IU x 1.07639 = MU | Unit (100 square meters) |
| Unit (100 square feet) | IU x 1 = MU | Unit (9.2 square meters) |
| Unit (100 square feet) | IU x 1.07639 = MU | Unit (10 square meters) |
| Board-foot | IU x 0.002359737 = MU | Cubic meter, m3 |
| 1,000 foot-board-measure, MFBM | IU x 2.359737 = MU | Cubic meter, m3 |
| VOLUME | ============= | VOLUME |
| Cubic foot, ft3 | IU x 0.02831685 = MU | Cubic meter, m3 |
| Cubic Inch, in3 | IU x 16.38706 = MU | Cubic centimeter, cm3 |
| Cubic yard, yd3 | IU x 0.7645549= MU | Cubic meter, m3 |
| VOLUME (U.S. LIQUID) | ============= | VOLUME (LIQUID) |
| Fluid ounce, fl oz | IU x 29.57353 = MU | Milliliter, ml |
| Gallon, gal | IU x 3.7854 = MU | Liter, l |
| Ounce, oz | IU x 29.57353 = MU | Milliliter, ml |
| Pint, pt | IU x 0.47318 = MU | Liter, l |
| Pint, pt | IU x 473.1765 = MU | Milliliter, ml |
| Quart, qt | IU x 0.94635 = MU | Liter, l |
| Unit (1,000 gallons) | IU x 1 = MU | Unit (3,785 liters) |
| MASS (WEIGHT) | ============= | MASS |
| Ounce, oz | IU x 28.34952 = MU | Gram, g |
| Ounce, oz | IU x 0.02834952 = MU | Kilogram, kg |
| Pound, lb | IU x 0.45359 = MU | Kilogram, kg |
| Ton (Short=2000 lbs) | IU x 0.9071847 =MU | Metric ton |
| Ton (Short=2000 lbs) | IU x 907.1847 =MU | Kilogram, kg |
| Tonnage | IU x 0.9071847 =MU | Metric tonnage |
| Weight | Equivalent Terms | Mass |
| MASS (APPLIED RATE) | ============= | MASS (APPLIED RATE) |
| Pounds per linear foot, lb/ft | IU x 0.45359 x 3.2808399 = MU | Kilograms per meter, kg/m |
| Pounds per linear foot, lb/ft | IU x 453.5924 x 3.28084 = MU | Grams per meter, g/m |
| Tons per inch | IU x 907.1847 x .03937 = MU | Kilograms per millimeter, kg/mm |
| DENSITY | ============= | DENSITY |
| Pounds per cubic foot, lb/ft3 | IU x 0.45359 x 35.314662 = MU | Kilograms per cubic meter, kg/m3 |
| Pounds per cubic yard, lb/yd3 | IU x 0.45359 x 1.3079505 = MU | Kilograms per cubic meter, kg/m3 |
| Pounds per gallon, lb/gal | IU x 453.5924 x 0.264172 = MU | Grams per liter, g/l |
| Pounds per gallon, lb/gal | IU x 0.45359 x 0.264172 = MU | Kilograms per liter, kg/l |
| Pounds per gallon, lb/gal | IU x 0.45359 x .264172 = MU | Kilograms per liter, kg/l |
| FORCE | ============= | FORCE |
| Kip (1000 pound-force, lbf) | IU x 4.448222 = MU | Kilonewton, kN |
| Pound, lbf | IU x 4.448222 = MU | Newton, N |
| Pound, lbf | IU x 0.004448222= MU | Kilonewton, kN |
| Poundal | IU x 0.1382550 = MU | Newton, N |
| Pounds per inch, lb/in | IU x 0.00445 x 39.37008 = MU | Kilonewtons per meter, kN/m |
| Pounds per linear foot, lb/lf | IU x 0.0044482 x 3.28084= MU | Kilonewtons per meter, kN/m |
| Ton (2000 pound-force, lbf) | IU x 8896.444 =MU | Newton, N |
| PRESSURE | ============= | PRESSURE |
| Ounces per square foot, oz/ft2 | IU x 28.34952 x 10.76391 = MU | Grams per square meter, g/m2 |
| Ounces per square yard, oz/yd2 | IU x 0.028349 x 1.19599 = MU | Kilograms per square meter, kg/m2 |
| Pounds per square foot, lb/ft2 | IU x 4.882428 x 10.7639 = MU | Kilograms per square meter, kg/m2 |
| Pounds per square inch, lb/in2 | IU x 0.006894757 = MU | Megapascals, MPa |
| Pounds per square inch, lb/in2 | IU x 6.894757 = MU | Kilopascals, kPa |
| Pounds per square yard, lb/yd2 | IU x 0.0004536 x 11959.9 = MU | Metric tons per hectare, |
| Pounds per square yard, lb/yd2 | IU x 0.45359 = MU | Kilograms per square meter, kg/m2 |
| WORK | ============= | WORK |
| Foot poundal | IU x 0.04214011 = MU | Joule (J) |
| Foot-pound, ft-lbf | IU x 0.3048 x 4.448222 = MU | Newtonmeters, Nm |
| Foot-pound, ft-lbf | IU x 1.355818 = MU | Joule (J) |
| Inch-pound, in-lbf | IU x .08333 x 1.355818 = MU | Joule (J) |
| Watt-hour, Wh | IU x 0.0036 = MU | Joule (J) |
| Kilowatt-hour, kWh | IU x 0.0000036 = MU | Joule (J) |
| WORK RATE | ============= | WORK RATE |
| Foot-pounds (25) at +10°F | Equivalent | Joules (34) at -12°C |
| Foot-pounds per inch (ft-lbf/in) | IU x 1.355818 x 39.37007 = MU | Joules per meter, J/m |
| Foot-pounds per inch (ft-lbf/in) | IU x 1.355818 x 39.37007 = MU | Newtonmeters per meter, Nm/m |
| Foot-pounds per pound (ft-lbf/lbf) | IU x 1.355818 x 4.448222 = MU | Joules per newton, J/n |
| Kips per foot | IU x 4.448222 x 3.28084= MU | Kilonewtons per meter, kN/m |
| Kips per square inch | IU x 4.448222 x 0.155 = MU | Kilonewtons per square centimeter, kN/cm2 |
| POWER | ============= | POWER |
| Foot-pound per second, ft-lbf/s | IU x 1.355818 = MU | Joule per second, J/s |
| Watt, W | IU x 1 = MU | Joule per second, J/s |
| FLOW | ============= | FLOW |
| Cubic feet per second, cfs | IU x 0.02831685 = MU | Cubic meters per Second, m3//s |
| Gallons per minute, gpm | IU x 0.0000630902 = MU | Cubic meters per Second, m3//s |
| Gallons per minute, gpm | IU x 3.7854 x 0.016667 = MU | Liters per second, l/s |
| Gallons per minute per foot , gpm/ft | IU x 3.7854 x 3.2808399 = MU | Liters per minute per meter, l/m/m |
| FLOW (PER OPENING AREA) | ============= | FLOW (PER OPENING AREA) |
| Cubic Feet per Second per square foot, cfs/ft2 | IU x 0.02831685 x 10.76394 = MU | Cubic meters per second per square meter, m3//s/ m2 |
| VELOCITY | ============= | VELOCITY |
| Feet per second, ft/s | IU x .3048 = MU | Meters per second, m/s |
| Miles per hour, mph | IU x 1.60934 = MU | Kilometers per hour, kph |
| ACCELERATION | ============= | ACCELERATION |
| Feet per second per second, ft/s/s | IU x .3048 = MU | Meters per second per second, m/s/s |
| TEMPERATURE | ============= | TEMPERATURE |
| Fahrenheit, °F | (IU minus 32) x 0.5556 = MU | Celsius, °C |
| COEFFICIENT OF EXPANSION |  | COEFFICIENT OF EXPANSION |
| Foot per degree F, ft/°F | IU x .3048 x 1.8 = MU | Meters per degree C, m/°C |
| LIGHT | ============= | LIGHT |
| Foot-candle | IU x 10.76391 = MU | Lux, lx |
| LIGHT (EXPOSURE RATE) | ============= | LIGHT (EXPOSURE RATE) |
| Candelas (or Candles) per foot-candle per square foot | IU x 10.76391 x 10.76391 = MU | Candelas (or Candles) per lux per square meter, cd/lx/m2 |
| Millicandelas per square foot per foot-candle | IU x 10.76391 x 10.7639 = MU | Millicandelas per square meter per lux, mcd/m2/lx |
| Millicandles (100) per square foot per foot-candle | IU x 10.76391 x 10.7639 = MU | Millicandles (100) per square meter per lux, 100mcd/m2/lx |
| STANDARD SIZES | ============= | STANDARD SIZES |
| Class A3 Concrete | Equivalent | Class 20 Concrete |
| Class A3.5 Concrete | Equivalent | Class 25 Concrete |
| Class A4 Concrete | Equivalent | Class 30 Concrete |
| Class A4.5 Concrete | Equivalent | Class 35 Concrete |
| Class A5 Concrete | Equivalent | Class 35-60 by 5’s Concrete |
| Class B2 Concrete | Equivalent | Class 15 Concrete |
| Class C1 Concrete | Equivalent | Class 10 Concrete |
| Class T3 Concrete | Equivalent | Class 20T Concrete |
| Gage, 06 (Gauge) (Diameter) | Equivalent | 4.88 Millimeter |
| Gage, 09 (Gauge) (Diameter) | Equivalent | 3.76 Millimeter |
| Gage, 12 (Gauge) (Diameter) | Equivalent | 2.67 Millimeter |
| Gage, 07 (Gauge) (Thickness) | Equivalent | 4.5 Millimeter |
| Gage, 10 (Gauge) (Thickness) | Equivalent | 3 Millimeter |
| Gage, 12 (Gauge) (Thickness) | Equivalent | 2.8 Millimeter |
| Gage, 13 (Gauge) (Thickness) | Equivalent | 2.32 Millimeter |
| Gage, 14 (Gauge) (Diameter) | Equivalent | 2.0 Millimeter |
| Gage, 16 (Gauge) (Thickness) | Equivalent | 1.55 Millimeter |
| Gage, 18 (Gauge) (Thickness) | Equivalent | 1.3 Millimeter |
| Gage, 20 (Gauge) (Thickness) | Equivalent | 0.9 Millimeter |
| Inch (Bolts) | Equivalent | See specific metric replacements |
| Sieve, 1 inch | Equivalent | Sieve, 25 millimeter |
| Sieve, 1/2 inch | Equivalent | Sieve, 12.5 millimeter |
| Sieve, 1-1/2 inch | Equivalent | Sieve, 37.5 millimeter |
| Sieve, 2 inch | Equivalent | Sieve, 50 millimeter |
| Sieve, 2-1/2 inch | Equivalent | Sieve, 63.5 millimeter |
| Sieve, 3 inch | Equivalent | Sieve, 75 millimeter |
| Sieve, 3/4 inch | Equivalent | Sieve, 19 millimeter |
| Sieve, 3/8 inch | Equivalent | Sieve, 9.5 millimeter |
| Sieve, 3-1/2 inch | Equivalent | Sieve, 90 millimeter |
| Sieve, 4 inch | Equivalent | Sieve, 100 millimeter |
| Sieve, No. 04 | Equivalent | Sieve, 4.75 millimeter |
| Sieve, No. 08 | Equivalent | Sieve, 2.36 millimeter |
| Sieve, No. 100 | Equivalent | Sieve, 150µm [aka micrometer, micron (.001mm)] |
| Sieve, No. 100 | Equivalent | Sieve, 150µm [aka micrometer, micron (.001mm)] |
| Sieve, No. 16 | Equivalent | Sieve, 1.18 millimeter |
| Sieve, No. 20 | Equivalent | Sieve, 850µm [aka micrometer, micron (.001mm)] |
| Sieve, No. 200 | Equivalent | Sieve, 75 millimeter |
| Sieve, No. 200 | Equivalent | Sieve, 75µm [aka micrometer, micron (.001mm)] |
| Sieve, No. 30 | Equivalent | Sieve, 600 millimeter |
| Sieve, No. 300 | Equivalent | Sieve, 270µm [aka micrometer, micron (.001mm)] |
| Sieve, No. 40 | Equivalent | Sieve, 425µm [aka micrometer, micron (.001mm)] |
| Sieve, No. 50 | Equivalent | Sieve, 300 millimeter |
| Sieve, No. 60 | Equivalent | Sieve, 250µm [aka micrometer, micron (.001mm)] |
| Sieve, No.10 | Equivalent | Sieve, 2.00 millimeter |
| IMPACT STRENGTH | ============= | IMPACT STRENGTH |
| Pounds (force) per inch, lbf/in | IU x 4.448222 x .03937 = MU | Newtons per millimeter, N/mm |
| ABSORPTION RATE | ============= | ABSORPTION RATE |
| Liters per mil | IU x 39.37 = MU | Liters per millimeter, l/mm |
| APPLICATION RATES | ============= | APPLICATION RATES |
| Gallons per cubic yard, gal/yd3 | IU x 3.7854 x 0.764555 = MU | Liters per cubic meter, l/m3 |
| Gallons per square foot, gal/ft2 | IU x 3.7854 x 10.76391 = MU | Liters per square meter, l/m2 |
| Gallons per square yard, gal/yd2 | IU x 3.7854 x 1.19598999 = MU | Liters per square meter, l/m2 |
| Gallons per ton | IU x 3.7854 x 0.9071847 = MU | Liters per metric ton |
| Inch per second per 0.1 inch | IU x 25.4 x 393.7007 = MU | Millimeters per second per meter, mm/s/m |
| Pounds per 100 square foot | IU x 0.45359 x 1 = MU | Kilograms per 9.29 square meters |
| Pounds per acre, lb/ac | IU x 0.45359 x 2.47104 = MU | Kilograms per hectare, kg/ha |
| Pounds per square foot per hour, lb/ft2/h | IU x 4.88243 x 10.76391 = MU | Kilograms per square meter per hour, kg/m2/h |
| Pounds per square yard per inch, lb/yd2/in | IU x 0.45359 x 0.8361274 = MU | Kilograms per square meter (25-millimeter nominal application rate) |
| Pounds per square yard per inch, lb/yd2/in | IU x 0.4536 x 0.8361 x 0.03937 = MU | Kilograms per square meter per millimeter, kg/m2/mm |
| Square foot per gallon, ft2/gal | IU x 0.092903 x 0.264172 = MU | Square meters per liter, m2/l |
| Tons per acre | IU x 0.9071847 x 2.47104 = MU | Metric tons per hectare |

III.	SPECIFIC METRIC REPLACEMENTS

The Virginia Department of Transportation Road and Bridge Specifications, dated 2016; its accompanying revisions including all Supplemental Specifications, Special Provisions, and Special Provision Copied Notes; the Virginia Department of Transportation Road and Bridge Standards, dated 2016; its accompanying revisions including all Standard sheets and Special Design drawings; the 2011 edition of the Virginia Work Area Protection Manual with Revision Number 1 incorporated, dated April 1, 2015; the 2009 edition of the MUTCD with Revision Numbers 1 and 2 incorporated, dated May 2012; and the 2011 edition of the Virginia Supplement to the MUTCD with Revision Number 1 dated September 30, 2013; and other contract documents included in the Contract contain specific elements such as tables, nomographs and forms that, as they appear in those imperial-only unit documents, are difficult to convert to metric units using only a conversion method.  The following are metric unit replacements for specific imperial units in this Contract:

A.	The VIRGINIA DEPARTMENT OF TRANSPORTATION ROAD AND BRIDGE SPECIFICATIONS book, dated 2016

Table III-2, Cold Weather Paving Limitatations

The imperial-only unit nomograph titled “Table III-2, Cold Weather Paving Limitatations” in the Virginia Department of Transportation Road and Bridge Specifications book, dated 2016 is replaced with the following:

Section 226.02(h)3 High-Strength Bolts, Nuts, Washers, and Direct Tension Indicators

1.15 x Proof Load (kN) and Max. Torque 1.15 x Proof Load (Nm) tables in the Virginia Test Method 135 are replaced with the following:

During this test, the minimum recorded tension shall be at least 1.15 times the required bolt proof load as specified in ASTM A325 and A490:

| 1.15 x Proof Load (kN) |  |  |
| --- | --- | --- |
| Bolt Size | 325 | A490 |
| M16x2 | 108.3 | 149 |
| M20x2.5 | 169 | 233 |
| M22x2.5 | 209 | 288 |
| M24x3 | 244 | 337 |
| M27x3 | 316 | 438 |
| M30x3.5 | 387 | 536 |
| M36x4 | 563 | 780 |

The measured torque to produce the required fastener tension shall not exceed the value obtained by the following equation:

Torque = 0.25 PD

Where:

Torque = measured torque (newtonmeters)

P = measured bolt tension (newtons)

D = nominal diameter (meters).

| Max. Torque 1.15 x Proof Load (Nm) |  |  |
| --- | --- | --- |
| Bolt Size | A325 | A490 |
| M16x2 | 430 | 600 |
| M20x2.5 | 850 | 1170 |
| M22x2.5 | 1150 | 1580 |
| M24x3 | 1460 | 2020 |
| M27x3 | 2130 | 2960 |
| M30x3.5 | 2900 | 4020 |
| M36x4 | 5070 | 7020 |

Section 403.07(c)1 for “Wave Equation Analysis”

The form for Wave Equation Analysis referenced in Section 403.07(c)1 of the Specifications titled “Pile and Driving Equipment Data Form” is replaced with the following:

| VIRGINIA DEPARTMENT OF TRANSPORTATION PILE AND DRIVING EQUIPMENT DATA FORM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pile Driving Contractor Or Subcontractor (Driven by): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | HAMMER | Manufacture: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Model: |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Rated Energy: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | @ |  |  |  |  |  |  |  |  |  |  |  | Length of Stroke |  |  |  |  |  |
|  |  |  |  | Modifications: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | RAM | Ram Mass: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ANVIL OR | Mass: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BASE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | STRIKER | Material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PLATE | Thickness: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Area: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Modulus of Elasticity - E: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (kpa) |  |
|  |  |  |  | Coefficient of Restitution: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CAP | Material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BLOCK | Thickness: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Area: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Modulus of Elasticity - E: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (kpa) |  |
|  |  |  |  | Coefficient of Restitution: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PILE CAP | Helmet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mass: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Bonnet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Materials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Anvil Block |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Drive head |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Accessories |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CUSHION | Cushion Material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Thickness: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Area: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Modulus of Elasticity - E: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (kpa) |  |
|  |  |  |  | Coefficient of Restitution: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PILE | Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Pile Size: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Length: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Diameter: |  |  |  |  |  |  |  |  |
|  |  |  |  | Cross Sectional Area: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Material: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mass/m: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Design Pile Capacity: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | metric tons |  |  |  |  |  |  |
|  |  |  |  | Description of Splice: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Tip Treatment Description: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Submitted By: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Company: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Phone: |  |  |  |  |  |  |  |  |  |  |  |
| Cc: | District Bridge Engineer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | State Bridge Engineer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Construction Manager |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B.	The VIRGINIA DEPARTMENT OF TRANSPORTATION ROAD AND BRIDGE STANDARDS, dated 2016

Appendix A-2, Standard Reinforcing Bars

Where imperial units are used in the Contract to specify Standard Reinforcing Bars, the following conversion chart shall apply:

| STANDARD REINFORCING BARS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMPERIAL |  |  |  |  | METRIC |  |  |  |
| Bar Size Designation | Diameter (in) | Area (in2) | Weight (lb/ft) |  | Bar Size Designation | Diameter (mm) | Area (mm2) | Mass (kg/m) |
| #3 | 0.375 | 0.11 | 0.376 |  | #10 | 9.5 | 71 | 0.56 |
| #4 | 0.5 | 0.2 | 0.668 |  | #13 | 12.7 | 129 | 0.994 |
| #5 | 0.625 | 0.31 | 1.043 |  | #16 | 15.9 | 199 | 1.552 |
| #6 | 0.75 | 0.44 | 1.502 |  | #19 | 19.1 | 284 | 2.235 |
| #7 | 0.875 | 0.6 | 2.044 |  | #22 | 22.2 | 387 | 3.042 |
| #8 | 1 | 0.79 | 2.67 |  | #25 | 25.4 | 510 | 3.973 |
| #9 | 1.128 | 1 | 3.4 |  | #29 | 28.7 | 645 | 5.06 |
| #10 | 1.27 | 1.27 | 4.303 |  | #32 | 32.3 | 819 | 6.404 |
| #11 | 1.41 | 1.56 | 5.313 |  | #36 | 35.8 | 1006 | 7.907 |
| #14 | 1.693 | 2.25 | 7.65 |  | #43 | 43 | 1452 | 11.38 |
| #18 | 2.257 | 4 | 13.6 |  | #57 | 57.3 | 2581 | 20.24 |