GUIDELINES – Asphalt resurfacing (surface treatment, slurry seal and latex only). {2007-c704bm1}

cn704-000110-00	COVERING CLEANING AND INSPECTING EXISTING RAISED PAVEMENT MARKERS  — The Contractor shall cover all existing raised pavement markers with a non-stick covering.  The Contractor shall ensure that no resurfacing material, duct tape, or adhesive comes into contact with the retroreflector.  The covering shall extend to include an area of 12 inches in front and in back of the casting, and the entire width of the casting.

After completion of the resurfacing operation, the covering shall be removed.  If the existing raised pavement marker retroreflectors are dirtied during paving operations (including dirtying from adhesive residue), they shall be fully cleaned or replaced by the Contractor to ensure minimum retroreflectivity as defined in Section 235 of the Specifications.  Any raised markers (including retroreflectors and/or castings) damaged by the Contractor’s operations shall be replaced by the Contractor and properly disposed of at no expense to the Department.  Replacement castings shall not be placed in the same location as the existing castings.  The void left by the dislodged casting shall be repaired according to the Specifications, and the replacement raised pavement marker properly installed in a new location at least 3 inches from the repair.

The covering, cleaning, and inspection of the raised markers will not be measured for payment.  All cost for performing this work shall be included in the price bid for other items of work.

9-17-15c; Reissued 7-12-16 (SPCN)