cq314-000100-00

GUIDELINES – For Richmond district projects with asphalt surface treatment. {2007-cu314000a}

VACUUMING ASPHALT SURFACE TREATMENTS

The Special Provision for ASPHALT SURFACE TREATMENT (SP314-000100-00) is amended as follows:

Section IV. (b) Modified Single Seal and Modified Double Seal Treatments is amended to include the following as the third paragraph:

All routes receiving an Asphalt Surface Treatment (Modified Single Seal or Modified Double Seal) shall be lightly broomed to remove loose aggregate prior to opening the pavement surface to traffic.  All excess aggregate material on the pavement surface travel lane(s), and accumulated along curb and gutter sections, the edge of pavement/lawns, and hard-surfaced driveway entrances shall be removed by mobile vacuum unit after surface treatment has been exposed to traffic as directed by the Engineer up to three weeks after final treatment has been opened to traffic.

Section VI. - Measurement and Payment is amended to include the following:

Vacuuming shall be included in the price of other appropriate items.

9-4-14; Reissued 7-12-16_(SPCN)