cq512-000310-00 [formerly cq512-030100-00]

GUIDELINES – All projects with guardrail and impact attenuators.{2007-SS51203}

SECTION 512—MAINTAINING TRAFFIC is amended as follows:

Section 512.03(h)1 Guardrail barrier service and terminal treatments is amended to replace the last sentence with the following:

All end terminals used in conjunction with guardrail barrier service shall be from manufacturers on the Materials Division’s Approved Products List No. 12 and the VDOT NCHRP 350 or MASH approved list linked in List No. 12.  New Guardrail Terminals (GR-7 & GR-9) shall be permanently identified in a location readily visible for inspection that is not susceptible to damage by stamping or engraving.  The identification shall include Manufacturer, Date and Site of Manufacture, and Model Number.

Section 512.03(i) Impact Attenuator Service, the first paragraph, is amended to replace the last sentence with the following:

New impact attenuators shall be permanently identified in a location readily visible for inspection that is not susceptible to damage by stamping or engraving.  The identification shall include Manufacturer, Date and Site of Manufacture, and Model Number.

Section 512.03(r) Truck-mounted or trailer-mounted attenuators, the second paragraph, is amended to replace the last sentence with the following:

New truck-mounted and trailer-mounted attenuators shall be permanently identified in a location readily visible for inspection that is not susceptible to damage by stamping or engraving.  The identification shall include Manufacturer, Date and Site of Manufacture, and Model Number.

3-18-16; Reissued 7-12-16_(SPCN)