cq515-000100-00

Guidelines: Use when requested by the District Maintenance Engineer.

PLANING OR MILLING PAVEMENT OF UNCERTAIN THICKNESS – If, due to the existing thin pavement structure and depth of the planning operation (except Trench Widening), the underlying soils or foundation aggregate is exposed, the Contractor shall immediately halt his operations and contact the Engineer to determine the immediate and potential condition of the roadway or roadway shoulder.  If the Engineer determines that the immediate or potential condition of the roadway or roadway shoulder is or will become unacceptable or unsafe, the Contractor shall submit a plan to pave back all sections of the roadway or roadway shoulder within 24 hours from when they were planed or milled at no additional cost to the Department.  The Contractor will not be allowed to resume his operations until this plan is accepted by the Engineer, and the Contractor is prepared to execute it.  If the Contractor fails to submit a plan within 24 hours of exposing underlying soils or foundation aggregates, the Engineer may conduct the remaining milling and paving for that route in accordance with Section 105.14(e) of the Specifications.

10-24-17 (SPCN)